# Daily Progress Report

| Installation / Location | PS-3 |
| --- | --- |
| Subject | Corrosion Monitoring Activities |
| Contract Number | LC241047A0 |
| Assigned Axess Representatives | Stuart Dunthorne |
| Date of Issue | 17-07-2025 |

## Activities Summary

Work activities continued today, with Coupon and ER Probe locations being serviced on PS3-B Upper Deck and Main Deck. Further details as follows.

| Time | Activities |
| --- | --- |
| 06:00 to 07:00 | Awaiting gas test & permit issue |
| 07:00 to 11:30 | Replace ER Probes at 2 locations and attempt retrieval at 1 other ER Probe location. Retrieve & replace coupons at 1 location |
| 11:30 to 12:30 | Lunch |
| 12:30 to 16:30 | Retrieve & replace coupons at 2 locations and external service of 1 ER Probe location. Move equipment to PS3-D |
| 16:30 to 18:00 | Reporting |

Locations Serviced Summary

| Tag No | Line Description | Work Completed |
| --- | --- | --- |
| 53-XCP-972 | 12-P-532221-1144 12” Gas Line from V-5366 MP Separator to E-4302 | Removed old ER Probe and installed new 5” T10 Cylindrical ER Probe. All pressure retaining seals replaced. Probe reading obtained |
| 53-XCP-1132 | 6-GP-532095-15150 6” HP Gas from Arab-C BH27 WHJ to PS3-B Production Manifold | Removed old ER Probe and installed new 6.5” T10 Cylindrical ER Probe. All pressure retaining seals replaced. Probe reading obtained |
| 53-XCC-945 | 12-OC-530093-15144X 12” Oil Line from Production Manifold to Test Header | 3” Strip Coupons Removed & Replaced. All pressure retaining seals replaced |
| 53-XCP-946 | 12-OC-530093-15144X 12” Oil Line from Production Manifold to Test Header | Attempted to replace the old ER Probe but steel structure obstructs retrieval tool pressure gauge/bleed assembly |
| 53-XCC-1430 | 12-OC-530234-15151 12” Oil Inlet to V-5307 | 3” Strip Coupons Removed & Replaced. All pressure retaining seals replaced |
| 53-XCP-1429 | 12-OC-530234-15151 12” Oil Inlet to V-5307 | Cap removed, threads brushed and re-greased |
| 53-XCC-1001 | 14-OC-530274-31441M | 3” Strip Coupons Removed & Replaced. All pressure retaining seals replaced |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | New Probe Size / Specification | Probe Readings | Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCP-972 | 12-P-532221-1144 12” Gas Line from V-5366 MP Separator to E-4302 | 150psi | 9 o Clock | Install New ER Probe | 5” Cylindrical T10 4500-T10-K03005-5.00-0-0 | Check – 769 Div – 96.4 M.Loss – 0.481 CR – N/A | 17-07-2025 | 25” | Access OK | None Required |
| Access Fitting Picture |  |  |  | Removed Device |  |  | New Probe |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | New Probe Size / Specification | Probe Readings | Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCP-1132 | 6-GP-532095-15150 6” HP Gas from Arab-C BH27 WHJ to PS3-B Production Manifold | 1500psi | 12 o Clock | Install New ER Probe | 6.5” Cylindrical T10 4500-T10-K03005-6.50.00-0-0 | Check – 755 Div – 74.4 M.Loss – 0.372 CR – N/A | 17-07-2025 | 25” | Access OK | None Required |
| Access Fitting Picture |  |  |  | Removed Device |  |  | New Probe |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Coupons ID / Holder Length | Coupon Appearance | Installed Coupons ID / Weight / Material / Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCC-945 | 12-OC-530093-15144X 12” Oil Line from Production Manifold to Test Header | 100psi | 12 o Clock | Replace 3” Strip Coupons | MQ047 & MQ048 12.25” Holder | Dark in colour Some soft sludge | NF801 – 37.0915 NF802 – 37.0069 C1018 17-07-2025 | 25” | Access OK Fitting under grating | None Required |
| Access Fitting Photograph |  |  |  | Retrieved Coupons / Device Photograph |  |  | Installed Coupons / Device Photograph |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | New Probe Size / Specification | Probe Readings | Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCP-946 | 12-OC-530093-15144X 12” Oil Line from Production Manifold to Test Header | 100psi | 12 o Clock | Install New ER Probe | N/A – Probe could not be retrieved | Previously Obtained | 17-07-2025 | 25” | Access for tool obstructed Fitting under grating | Replace Probe during shutdown |
| Access Fitting Picture |  |  |  | Removed Device |  |  | New Probe |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |
| An attempt was made to retrieve the ER Probe at this location, but part of the platform structure obstructs the retrieval tool pressure gauge/bleed assembly so could not be retrieved. This location will need to be serviced during a shutdown. |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Coupons ID / Holder Length | Coupon Appearance | Installed Coupons ID / Weight / Material / Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCC-1430 | 12-OC-530234-15151 12” Oil Inlet to V-5307 | 150psi | 12 o Clock | Replace 3” Strip Coupons | MQ033 & MQ044 12.25” Holder | Dark in colour Pitting on leading edges | NE618- 37.0399 NE619 – 36.6463 C1018 17-07-2025 | 25” | Access OK | None Required |
| Access Fitting Photograph |  |  |  | Retrieved Coupons / Device Photograph |  |  | Installed Coupons / Device Photograph |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Work Performed | Probe Readings | Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCP-1429 | 12-OC-530234-15151 12” Oil Inlet to V-5307 | N/A | 12 o Clock | External Service of Access Fitting | Cap removed, threads brushed and re-greased | Obtained Previously | 17-07-2025 | N/A | Access OK | None Required |
| Access Fitting Picture |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Coupons ID / Holder Length | Coupon Appearance | Installed Coupons ID / Weight / Material / Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCC-1001 | 14-OC-530274-31441M | 100psi | 12 o Clock | Replace 3” Strip Coupons | MQ037 & MQ038 13.50” Holder | Dark in colour Some soft sludges | NE620 – 36.9794 NE621 – 36.7801 C1018 17-07-2025 | 25” | Access OK | None Required |
| Access Fitting Photograph |  |  |  | Retrieved Coupons / Device Photograph |  |  | Installed Coupons / Device Photograph |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |

## Observations & Recommendations

## One of the ER Probe locations we had hoped to replace today on PS3-B Upper deck has a handrail obstruction. Arrangements have been made with the mechanical team to have the handrail removed to allow access for the tool.

24 Hour Lookahead

The plan for Friday 18th July is to complete the remaining work on PS3-B and hopefully complete all work on PS3-D

Stuart Dunthorne – Axess-Corrosion. – sdunthorne@axess.energy