# Daily Progress Report

| Installation / Location | PS-3 |
| --- | --- |
| Subject | Corrosion Monitoring Activities |
| Contract Number | LC241047A0 |
| Assigned Petromech Representatives | Shuaib Palakkavalappil |
| Date of Issue | 23-08-2025 |

## Activities Summary

Work activities commenced today at PS-3, starting with  Corrosion Coupon Retrieval at 1 location where reported obstruction in last survey.

ER probe replacement also started late in the afternoon, with 1 locations being serviced where there is scaffolding obstruction reported. Further details of the se locations can be found below.

| Time | Activities |
| --- | --- |
| 06:00 to 09:30 | Proceed to dock ,board vessel to PS-3. Make arrangement for work permit to be issued |
| 09:30 to 11:30 | Commence Coupon Retrievals |
| 11:30 to 12:30 | Lunch |
| 12:30 to 16:00 | Commence ER Probe Replacement |
| 16:30 to 18:00 | Proceed to dock ,board vessel to Halul |

Locations Serviced Summary

| Tag No | Line Description | Work Completed |
| --- | --- | --- |
| 53-XCC-909 | 20” Oil Line from MP Separator to V5367 Surge Vessel | Removed 3” Strip Coupons with holder size 10.75”.Replaced 20” Flush disc holder. Replaced all pressure retaining seals |
| 53-XCP-946 | 12” Oil Line from Production Manifold to Test Header | Replaced old ER Probe but steel structure obstructing the retrieval tool gauge, it should replace while perform retrieval activity. 25” RBS Retrieval tool used |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Coupons ID / Holder Length | Coupon Appearance | Installed Coupons ID / Weight / Material | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCC-909 | 20-OC-530266-1144 20” Oil Line from MP Separator to V5367 Surge Vessel | 150 | 12 o Clock | Remove & Replace Corrosion Coupons | MQ031 & MQ032 Holder – 10.25” Installed on | Dark in colour small number of soft deposits | XR911-17.4135g 20” Flush disc holder 23-08-2025 | 37” | Access Ok. | None Required |
| Access Fitting Photograph |  |  |  | Retrieved Coupons / Device Photograph |  |  | Installed Coupons / Device Photograph |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Probe deatails | Date | New Probe Siz/Specification | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCP-946 | 12-OC-530093-15144X 12” Oil Line from Production Manifold to Test Header | 100 | 12 o Clock | Remove & Replace ER Probe | 4500-T10-K3005-11.25-0-1 Installed date -29-05-2014 | 23-08-2025 | 11.25” size 4705-S8-K03005-11.25-0 | 25” | Access for tool obstructed fitting under grating | Remove the grating while performing the Retrieval |
| Access Fitting Photograph |  |  |  | Retrieved Coupons / Device Photograph |  |  | Installed Coupons / Device Photograph |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |
| Replaced old ER Probe but steel structure obstructing the retrieval tool gauge, it should replace while perform retrieval activity. 25” RBS Retrieval tool used |  |  |  |  |  |  |  |  |  |  |

## Observations & Recommendations

It was noticed during the ER Probe replacement  at the location 53-XCP-946 that the access for tool obstructed by grating, it should remove while performing retrieval and 25” RBS retrieval tool is most suitable for the location

24 Hour Lookahead

The plan for Sunday 24th August is to continue with coupon and probe replacements on PS-3C Platform.

Shuaib Palakkavalappil – Petromech – shuaib.p@petromech.com.qa