# Daily Progress Report

| Installation / Location | PS-3 |
| --- | --- |
| Subject | Corrosion Monitoring Activities |
| Contract Number | LC241047A0 |
| Assigned Petromech Representatives | Shuaib Palakkavalappil |
| Date of Issue | 25-08-2025 |

## Activities Summary

Work activities at PS-3 continued today, Due to Heat Index we started the work lately. Work commenced today, starting with Corrosion Coupon Retrieval at 1 location.

Corrosion Coupon Retrievals and ER Probe replacement also started late in the afternoon, with 2 locations being serviced. Further details of the se locations can be found below.

| Time | Activities |
| --- | --- |
| 06:00 to 10:30 | Proceed to dock, board vessel to PS-3.Make arrangements for work permits to be issued, Due to heat index permit issued lately |
| 10:30 to 11:30 | Commence Coupon Retrieval |
| 11:30 to 12:30 | Lunch |
| 12:30 to 16:30 | Continue ER Probe Replacement and Coupon Retrieval |
| 16:30 to 18:30 | Proceed to dock, board vessel to PS-3 |

Locations Serviced Summary

| Tag No | Line Description | Work Completed |
| --- | --- | --- |
| 53-XCC-1428 | 18-OCP-530235-15151 18” Crude inlet to V-5356 | Removed & Replaced 3” Strip Coupons. Retrieved 12.25” strip holder and Replaced 16.5” strip holder. Replaced all pressure retaining seals |
| 53-XCP-1427 | 18-OC-530235-15151 18” Crude inlet to V-5356 | Probe location Serviced, Removed old 13.5” Type :B ER Probe and Replaced 17.75” Type :D ER Probe |
| Not Available | 6-GC-430036 | Protective cover jammed |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Coupons ID / Holder Length | Coupon Appearance | Installed Coupons ID / Weight / Material | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCC-1428 | 18-OCP-530235-15151 18” Crude inlet to V-5356 | 100psi | 12 o Clock | Remove & Replace Corrosion Coupons | MQ035 & MQ036 Holder – 12.25” | Dark in colour small amount of soft deposits | NE652 – 36.7398g NE653 – 36.8152g Holder-16.5” 25-08-2025 | 37”RBS Retrieval Tool | Not Accessible Elevated Level | Permanent Service platform Required |
| Access Fitting Photograph |  |  |  | Retrieved Coupons / Device Photograph |  |  | Installed Coupons / Device Photograph |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |
| Access at Elevated Level, Permanent Service Platform Required |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Coupons ID / Holder Length | Installed Coupons ID / Weight / Material | Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Not Available | 6-GC-430036 | 200 | 6 o Clock | Remove & Replace Corrosion Coupons | N/A- Protective cover Jammed | N/A | 25-08-2025 | N/A | Access Elevated level | Scaffolding Required |
|  |  |  |  | Access Fitting Picture |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |
| Access at Elevated Level, Scaffolding Required. Protective cover Jammed |  |  |  |  |  |  |  |  |  |  |

| Tag No | Line No / Description | Pressure (psi) | A/F Orientation | Scope of Work | Retrieved Probe Type/ S/N | Installed Probe Type / Reading | Installed Date | Tool Used | Location Access | Remedial Actions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53-XCP-1427 | 18-OC-530235-15151 18” Crude inlet to V-5356 | 100 | 12 o Clock | Remove & Replace ER PROBE | 4705-S8-K03005-13.5”-1 Type:B Installed Date: 25-05-2019 | 4500-T10-K03005-17.75-0-0 Check -771 Div-76.8 | 25-08-2025 | 37” RBS Retrieval tool | Not Accessible Elevated Level | Permanent Service platform Required |
| Access Fitting Photograph |  |  |  | Retrieved Coupons / Device Photograph |  |  | Installed Coupons / Device Photograph |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |
| Access at Elevated Level, Permanent Service Platform Required |  |  |  |  |  |  |  |  |  |  |

## Observations & Recommendations

It was noticed during the ER Probe Service and Corrosion Coupon Replacement at 53-XCP-1427&53-XCC-1428 Respectively that both the access at elevated level ,Permanent Service Platform Required

24 Hour Lookahead

The pending works at PS-3 Completed by Today

Shuaib Palakkavalappil –Petromech. – shuaib.p@petromech.com.qa